Locking plates

  - Fixation of fractures in osteoporotic bone.

- Indirect fracture reduction & bridging severely comminuted fractures& produce less angulation in comminuted metaphyseal fractures.

  - Plating of fractures where anatomical constraints prevent plating on tension side of bone.

- locking screw biomechanics:

Bicortical locking screws have more resistance to all applied forces especially torsion (versus unicortical).

  - unicortical locking screws have less torsion strength than bicortical non-locking screws.

- percutaneous locking plates has less soft-tissue stripping but higher chance Malunion.

- locking screws have greater core diameter than conventional screws (bending stiffness proportional to 4th power of radius).

- Screws - plate function as a fixed angle device.

- The working length of a monocortical locked screw is the amount of screw in contact with the cortex (shorter in osteoporotic bone). It’s not dependent on distance of plate from the bone, nor diameter of the screw. Not dependent on number of screws and is the same for self-tapped and nonself-tapping screws.

- Hybrid locked plates (double working) utilize nonlocking screws to assist fracture reduction as well as provide a fixed angle construct by locking screws.

- Stability increases with: bicortical locking screws,,, increased number of screws (longer plate),,, Screw divergence from screw hole < 5 degrees,,,.

- Starts non locking screws for reduction & then locking screws for rigidity.

** Locking plates reduce interfragmentary strain more than non-locking plating.

** Not used in Oligotrophic nonunion.. Not used in transverse #s for compression.

** NO bending in locked plates. It cause screw mechanics disturbance.

** Upper humerus (femur) anatomical plate screws in groups for different screw directions.